Q1)

- /////////////// AND BEHAVIOURAL SCHEME

module andgate(a,b);

input [1:0] a;

output reg b;

always@(a)

begin

case(a)

2'b11:b=1'b1;

default:b=1'b0;

endcase

end

endmodule

//////////////// XOR BEHAVIOURAL SCHEME

module xorgate(a,b);

input [1:0] a;

output reg b;

always@(a)

begin

case(a)

2'b00:b=1'b0;

2'b11:b=1'b0;

default:b=1'b1;

endcase

end

endmodule

- /////////////// HALF ADDER STRUCTUAL MODELLING

module xor1(input a, b, output s);

xorgate (a, b,s);

endmodule

module and1(input a, b, output c);

andgate (a, b,c);

endmodule

module half_adder (input a, b, output s, c);

xor1 u1(a, b, s);		// s->sum

and1 u2(a, b, c);	// c->carry

endmodule

/////////////// TESTBENCH

module half_adder_verilog_tb();

reg a, b;

wire s, c;

half_adder dut (.a(a), .b(b), .s(s), .c(c));

initial

begin

a = 1'b0;

b = 1'b0;

#50;

a = 1'b0;

b = 1'b1;

#50;

a = 1'b1;

b = 1'b0;

#50;

a = 1'b1;

b = 1'b1;

end

endmodule

- ////////////////// FULL ADDER USING 2 HALF ADDERs AND 1 OR GATE

///////////////// OR GATE MODULE

module orgate(a,b,c);

output c;

input a,b;

assign c = (a | b);

endmodule

//////////////////  FULL ADDER MODULE

module full_adder( A, B, Cin, Sum, Cout );

input A;

input B;

input Cin;

output Sum;

output Cout;

wire ha1_sum;

wire ha2_sum;

wire ha1_carry;

wire ha2_carry;

wire Sum;

wire Cout;

//Instantiate the half adder 1

half_adder  ha1( .a(A), .b(B), .s(ha1_sum), .c(ha1_carry) );

//Instantiate the half adder 2

half_adder  ha2( .a(Cin), .b(ha1_sum), .s(ha2_sum), .c(ha2_carry) );

//sum output from 2nd half adder is connected to full adder output

assign Sum = ha2_sum;

//The carry's from both the half adders are OR'ed to get the final carry./

assign Cout = ha1_carry | ha2_carry;

endmodule

///////////////////////// TESTBENCH

module tb_fullAdd;

// Inputs

reg A;

reg B;

reg Cin;

// Outputs

wire Sum;

wire Cout;

// Instantiate the Unit Under Test (UUT)

full_adder uut ( .A(A), .B(B), .Cin(Cin), .Sum(Sum), .Cout(Cout) );

initial begin

//Apply inputs. 8 combinations of inputs are possible.

A = 0;  B = 0;  Cin = 0;  #100;

A = 0;  B = 0;  Cin = 1;  #100;

A = 0;  B = 1;  Cin = 0;  #100;

A = 0;  B = 1;  Cin = 1;  #100;

A = 1;  B = 0;  Cin = 0;  #100;

A = 1;  B = 0;  Cin = 1;  #100;

A = 1;  B = 1;  Cin = 0;  #100;

A = 1;  B = 1;  Cin = 1;  #100;

end

endmodule

//////////////////////////////////////////

Q2) //////////////// 4 BIT RIPPLE CARRY ADDER

module ripple_adder_4bit ( output [3:0] Sum, output Cout, input [3:0] A,B, input Cin );

wire c1,c2,c3;

full_adder FA1(Sum[0],c1,A[0],B[0],Cin),

FA2(Sum[1],c2,A[1],B[1],c1),

FA3(Sum[2],c3,A[2],B[2],c2),

FA4(Sum[3],Cout,A[3],B[3],c3);

endmodule

/////////////////Test Bench for 4 Bit Ripple Carry Adder

module test_ripple_adder_4bit;

reg [3:0] A;

reg [3:0] B;

reg Cin;

wire [3:0] Sum;

wire Cout;

ripple_adder_4bit uut ( .Sum(Sum), .Cout(Cout), .A(A), .B(B), .Cin(Cin) );

initial begin

A = 0;

B = 0;

Cin = 0;

#100;

A=4'b0001;B=4'b0000;Cin=1'b0;

#10 A=4'b1010;B=4'b0011;Cin=1'b0;

#10 A=4'b1101;B=4'b1010;Cin=1'b1;

end

initial begin

$monitor("time=",$time,, "A=%b B=%b Cin=%b : Sum=%b Cout=%b",A,B,Cin,Sum,Cout);

end

endmodule

//////////////////////////

Q3) ///////// FULL SUBTRACTOR USING GATE LEVEL PRIMITIVES

module fs(a, b, c, borrow, difference);

input a;

input b;

input c;

output borrow;

output difference;

wire d,e,f;

xor(difference,a,b,c);

and(d,~a,b);

and(e,b,c);

and(f,~a,c);

or(borrow,d,e,f);

endmodule

//////////////////// TESTBENCH

module fullsubt_b;

reg a;

reg b;

reg c;

wire borrow;

wire difference;

fs uut (.a(a),  .b(b),.c(c),.borrow(borrow),.difference(difference)  );

initial begin

#10 a=1’b0;b=1’b0;c=1’b0;

#10 a=1’b0;b=1’b0;c=1’b1;

#10 a=1’b0;b=1’b1;c=1’b0;

#10 a=1’b0;b=1’b1;c=1’b1;

#10 a=1’b1;b=1’b0;c=1’b0;

#10 a=1’b1;b=1’b0;c=1’b1;

#10 a=1’b1;b=1’b1;c=1’b0;

#10 a=1’b1;b=1’b1;c=1’b1;

#10$stop;

end

endmodule

Q4) //////////// 4BIT ADDER CUM SUBTRACTOR

module adder_subtactor_4bit ( a ,b ,sel ,dout );

output [3:0] dout ;

input [3:0] a ;

input [3:0] b ;

input sel ;

wire [3:0]m;

wire [3:0]n;

adder_4bit u0  (.a (a), .b (b), .sum (m));

subtractor_4bit u1 (.a(a), .b(b), .diff (n));

mux_4bit u2 (.a(m), .b(n), .sel(sel), .dout(dout));

endmodule

/////////// 4BIT ADDER

module adder_4bit ( a ,b ,sum ,carry );

output [3:0] sum ;

output carry ;

input [3:0] a ;

input [3:0] b ;

wire [2:0]s;

full_adder u0 (a[0],b[0],1'b0,sum[0],s[0]);

full_adder u1 (a[1],b[1],s[0],sum[1],s[1]);

full_adder u2 (a[2],b[2],s[1],sum[2],s[2]);

full_adder u3 (a[3],b[3],s[2],sum[3],carry);

endmodule

////////// 4BIT SUBTRACTOR

module subtractor_4bit ( a ,b ,diff ,borrow );

output [3:0] diff ;

output borrow ;

input [3:0] a ;

input [3:0] b ;

wire [2:0]s;

full_adder u0 (a[0],b[0],1'b1,diff[0],s[0]);

full_adder u1 (a[1],b[1],s[0],diff[1],s[1]);

full_adder u2 (a[2],b[2],s[1],diff[2],s[2]);

full_adder u3 (a[3],b[3],s[2],diff[3],borrow);

endmodule

//////////////// FULL ADDER

module full_adder ( a ,b ,c ,sum ,carry );

output sum ;

output carry ;

input a ;

input b ;

input c ;

assign sum = a ^ b ^ c;

assign carry = (a&b) | (b&c) | (c&a);

endmodule

////////////// 2:1 MUX

module mux_4bit ( a ,b , sel, dout );

output [3:0]dout ;

input [3:0]a ;

input [3:0]b ;

input sel ;

assign dout = sel ? b : a;

endmodule

/////////////// TESTBENCH

module add_sub_tb;

reg [3:0] a;

reg [3:0] b;

wire sel;

wire [3:0] dout;

adder_subtactor_4bit uut (.a(a),  .b(b),.sel(sel),.dout(dout) );

initial begin

#10 a=4’b0101;b=4’b0100;sel=1’b0;

#10 a=4’b0101;b=4’b0100;sel=1’b1;

#10 a=4’b1110;b=4’b0010;sel=1’b0;

#10 a=4’b1110;b=4’b0010;sel=1’b1;

#10 a=4’b1001;b=4’b0011;sel=1’b0;

#10 a=4’b1001;b=4’b0011;sel=1’b1;

#10$stop;

end

endmodule