Computer Basics, What is the Architecture?

Computers have become a way of life.  We use them to pay bills, play games, build bridges or communicate with friends and family.  The personal computer has become a staple component in nearly every aspect of our lives.  However, just like a toaster or cable box, many users don’t know the internal workings that store and process all the data we view.

There are many components that influence a computers performance.  The architecture is dependent on both hardware and software, but the vary basics can by identified by some fundamental concepts that have roots in the von Neumann architecture.

For a computer designer, understanding the von Neumann architecture is critical to understanding the design of the arithmetic unit that modern computers are based on and included basic controlling characteristics like the system bus, Boolean operators and various types of memory and storage.

# Von Neumann Architecture

John von Neumann’s contribution to science has influences in many different disciplines. However, possibly the greatest influence on computer architecture was Von Neumann’s stored program concept.  Prior to von Neumann’s theories, computers where special purpose, only capable of performing specific tasks.  Not until John von Neumann authored two papers in 1945 and 1946 did engineers change how computers where designed and von Neumann’s influence gave birth the first computer specifically designed for multiple use. This computer was called the “Electronic Discrete Variable Automatic Computer,” or EDVAC.  The EDVAC became the model for modern computers and is known as the Von Neumann machine.

Von Neumann’s’ design relied on four main parts: The Central Arithmetical Unit (CA or arithmetic logic unit (ALU)), the Central Control unit (CU), the Memory (M) and the Input/output devices (IO).  This architecture, called fetch-execute cycle, allows a computer the ability to store data, the results of the computation and store the instructions that produced the computation.

# System Bus

Like the human brain that uses neurons and transmitters to move information, the Central Processing Unit (CPU) needs a bridge to facilitate data transfer.  A modern computer has many components that communicate on different levels and require many different components and protocols for specific types of traffic.  This bridge, called a bus, can communicate with both internal and external devices, but the system bus provides the primary pathway between the CPU and memory, called random access memory (RAM).  Also known as the frontside bus, this bus is part of the motherboard chip set and other buses branch off of it.

The speed of a computer relies on many different factors.  The speed of the CPU, RAM and hardrive can all influence how fast traffic travels between components.  The system bus is designed to match the type of processor and increases in width and speed according to CPU speed and requires specific RAM.  Like a lake that is fed by rivers. The larger the river, the more water the lake can receive.  Modern motherboards, especially from AMD or Intel’s Rambus RAM technology, have moved away from the system bus architecture to increase bandwidth between the CPU and RAM.  This new bus interface is called HyperTransport, from AMD and Intel QuickPath

Interconnect for Intel.

# Boolean Operators

To understand the need for Boolean operators, one must first understand the language of computers.  Binary code is created as transistors are turned on or off.  This on and off transition is recorded as 1’s and 0’s and string of 1’s and 0’s is interpreted as letters, words and commands.

Three words, AND, OR and NOT, known as Boolean operators create a relationship between two inputs and the output.   Table 1 is a truth table and best visualizes the AND operator.

| Inputs | ¦ | Output |
| --- | --- | --- |
| X | Y | Z |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Figure 1

In the truth table, the first row in column X equals 0, AND the first row in column Y equals 0 will output 0 to create a Boolean expression. XY = Z.

Figure 2 shows the truth table for the OR operator.

| Inputs | ¦ | Output |
| --- | --- | --- |
| X | Y | Z |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Figure 2

The OR truth table shows that X OR Y results in Z.  The output Z will be a 1 unless both inputs are not a 1. X or Y result in Z can be represented as X+Y=Z.

The last truth table in figure 3 represents the NOT operator.

| Inputs | Output |
| --- | --- |
| X | Z |
| 0 | 1 |
| 1 | 0 |

Figure 3

As you can see, the input X=0 results in it NOT being 0.  As stated in Connecting with Computer Science, “Each basic Boolean operator can be combined with Boolean variables to create complex Boolean expressions.”

# Memory and Storage

Earlier, we alluded to the need for the system bus to create a bridge between the CPU and RAM.  In order for the CPU to fetch-execute, there are a couple of components needed.  Memory gives the CPU the ability to read and write, but storage is needed to keep programs and data for long term use.

Memory

There are several kinds of memory that comes in two basic types.  Read only memory (ROM) and RAM.  ROM is non-volatile, meaning that data is not lost when the power is removed.

ROM is also known as firmware and is an integrated circuit with permanent data stored on the chip.

This chip, called the basic input/output system (BIOS), stores instructions and data that provide startup programs and basic I/O routines.

Although ROM can’t be written to, special ROM types like programmable read-only memory (PROM), erasable programmable memory (EEPROM), electrically erasable programmable memory (EEPROM) are designed to be partially or wholly erased under specific conditions.

RAM, by contrast, is volatile memory that losses data when the power is turned off.  In order for the CPU to receive instructions, every program is loaded into RAM for the CPU to fetch and execute.  Through computer history, different types of RAM with varying speeds have been designed.  Dynamic RAM (DRAM), DDRRAM, synchronous DRAM (SDRAM) DDR2, DDR3,

and the new DDR4 provide faster speeds with each progression.

Synchronous RAM (SRAM) is a volatile RAM with much faster clock speeds than DRAM.

Because of its volatility, SRAM is used in CPU caches, hard disk buffers and LCD screens.

Storage

Storage also comes in a variety of sizes and uses.  Some storage devices use RAM technology, called flash drives, to store small amounts of data for portability.  However, mass storage devices are needed to hold our music, movies, games and files.  The cheapest and still most common mass storage device is the hard disc drive (HDD).  HDD’s come with metal platters coated with magnetic particles.

When the electromagnetic recording head moves from one direction to the other, binary code is written to the platter.  Although the platters can spin very fast, (The most common platter spins at 7200 RPM), they are slow in comparison to solid state drives (SSD).  However, the price per gigabyte is much cheaper with HHD’s than SSD’s and come in much larger capacities. A common configuration might include an SSD with the operating system installed and an HHD for saving files.

It is easy to see how von Neumann’s contribution has created a revolution in technology not much different than the Industrial revolution did in the mid 1700’s.  Without his architectural ideas, we might still be using multiple devices for common computer uses.  A calculator for adding, a word processor for writing, an internet box for surfing the Internet or simple punch cards for recording work hours.  Most of us today take for granted how technology has taken a jump start from tubes and electrodes to motherboards with CPU’s, RAM and mass storage devices.

References

Allied Electronics. (2011, nd). About the system bus. Retrieved from Allied Electronics:

http://www.karbosguide.com/hardware/module2b2.htm

AMD. (nd). AMD HyperTransport technology. Retrieved from AMD:

http://www.amd.com/us/products/technologies/hypertransport-

technology/Pages/hypertransport-technology.aspx

Anderson, G., Ferro, D., & Hilton, R. (2011). Connecting with Computer Science (2nd ed.). Boston, Massachusetts: Cengage Learning.

Cabera, B. J. (nd). John von Neumann and von Neumann Architecture for Computers (1945).

Retrieved from Salem State College: http://w3.salemstate.edu/~tevans/VonNeuma.htm

Intel. (nd). Intel quickpath interconnect maximizes multi-cor performance. Retrieved from Intel architecture and silicone: http://www.intel.com/content/www/us/en/io/quickpathtechnology/quickpath-technology-general.html

Riley, N. (1987, September nd). The von Neumann Architecture of Computer Systems. Retrieved from Computer science department California State polytechnic university:

http://www.csupomona.edu/~hnriley/www/VonN.html

Tyson, J. (nd). How ROM works. Retrieved from How stuff works:

http://computer.howstuffworks.com/rom.htm

Powered by TCPDF (www.tcpdf.org)